Quiz 3: Thermodynamic property calculation for oxygen [Due for submission at 5 pm September 30, 2022]

Name: Aditya Narayan Prasad

Role number: 210020007

The Peng-Robinson equation of state is

| P=RTV-b-aTVV+b+bV-b | where |
| --- | --- |
| aT=0.45724R2Tc2Pc1+κ1-TTc 2 | b=0.0778RTcPc |

For oxygen

| κ=0.4069 | Tc=154.6 K | Pc=5.046 MPa |
| --- | --- | --- |

This EOS can be written in a cubic polynomial form as

Z3+αZ2+βZ+γ=0

where

| α=B-1 | β=A-3B2-2B | γ=B3+B2-AB |
| --- | --- | --- |
| A=aP/RT2 | B=bP/RT | Z=PV/RT. |

(a) Find the vapor pressure Pvap(T) at temperatures T=-125, -150 and -175 ℃.

Hint: To find the vapor pressure you require T and a guess value of P. Figure 6.4-3 can be used to obtain a guess value. Solve for ZL and ZV using the cubic equation Z3+αZ2+βZ+γ=0. We shall employ fugacity. For vapor, calculate the vapor fugacity fV with the help of

lnfVP=ZV-1-lnZV-B-A22BlnZV+1+2BZV+1-2B.

For liquid, calculate the liquid fugacity fL with the help of

lnfLP=ZL-1-lnZL-B-A22BlnZL+1+2BZL+1-2B.

Recall that A and B depend on T and P. Therefore, they have to be calculated each time for a new value of T and P.

For vapor-liquid coexistence we require fV(T,P)=fLT,P. Therefore, check whether fLfV-1<0.0001. If so, then you can stop – you have a solution for Pvap at the selected temperature. Otherwise, set P=PfLfV. The flowchart below will help:

Pvap(-125 C) = 3.9422 * 106 Pa

Pvap(-150 C) = 1.2258 * 106 Pa

Pvap(-175 C) = 2.1931 * 105 Pa

Code for Vapour Pressure as a Function Of Temperature

function y = p_vap(T)

Pc = 5.046e6;

Pvap = Pc; %Pa -- initial guess

nroots=1;

while nroots~=3 %check whether this initial guess provides 3 roots

z=PR(T,Pvap);

nroots=size(z,1);

if nroots==1

Pvap=Pvap-10e5; %New guess

end

end

fl = 1;

fv = 0.1;

while abs((fl/fv)-1) > 0.0001

[A, B] = pt_consts(T, Pvap);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

check_zr=abs(polyval(PReq,zr))<1.e-8 & zr>1.e-5;

zl = zr(3);

zv = zr(1);

fl = f(zl, T, Pvap);

fv = f(zv, T, Pvap);

Pvap = Pvap*fl/fv;

end

y = Pvap;

end

Function for Fugacity

function y = f(Z, T, P)

[A B] = pt_consts(T, P);

term1 = Z-1;

term2 = log(Z-B);

term3 = (Z + (1+sqrt(2))*B)/(Z + (1-sqrt(2))*B);

term3 = (A/(sqrt(8)*B))*log(term3);

y = exp(term1 - term2 - term3).*P;

end

b) Plot the isotherm P-V curve at temperatures T=-125, -150 and -175 ℃ as shown in Figure 6.4-3 (page 1).

P-V Plot for T = -125, -150 and -175 C

Code

- PV plot

R = 8.314;

v = logspace(-2, 1, 500)*1e-3;

p = zeros(1, size(v, 2));

t = [-175 -150 -125]+273.15;

for j =1:size(t, 2)

vapour_p = p_vap(t(1, j));

[A, B] = pt_consts(t(1, j), vapour_p);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

check_zr=abs(polyval(PReq,zr))<1.e-8 & zr>1.e-5;

zl = zr(3);

zv = zr(1);

vl = zl*R*t(1, j)/vapour_p;

vu = zv*R*t(1, j)/vapour_p;

for i = 1:size(v, 2)

if v(1, i) <vl

p(1, i) = pv(v(1, i), t(1, j));

elseif v(1, i) > vu

p(1, i) = pv(v(1, i), t(1, j));

else

p(1, i) = vapour_p;

end

end

semilogx(v, p);

xlim([2e-5 0.01]);

ylim([1e5 100e5]);

hold on;

end

xlabel('Volume (m^3/mol)');

ylabel('Pressure (Pa)');

legend('T = -175 C', 'T = -150 C', 'T = -125 C');

- PV

function y = pv(V, T)

Pc = 5.046e6;

Tc = 154.6;

R = 8.314;

b = 0.0778*(R*Tc)/Pc;

a = 0.45724*((R*Tc)^2)/Pc;

kappa = 0.4069;

alpha=1+kappa*(1-sqrt(T/Tc));

alpha=alpha*alpha;

a=a*alpha;

term1 = (R*T)/(V-b);

term2 = a/(V^2 + 2*V*b - b^2);

y = term1 - term2;

end

(c) Make a plot for Pvap versus T for the vapor-liquid coexistence. Choose temperatures T=-125, -130, -140, -150, -160, -170, -175, -180 and -183 ℃.

Pvap versus T

- For T=-125, -130, -140, -150, -160, -170, -175, -180 and -183 ℃, Pvap in Circles

- Best fit curve in red line gotten through lagrange interpolation

Code

Pvap for T = -180 to -125 C & Curve Fitting

T = [-183 -180 -175 -170 -160 -150 -140 -130 -125] + 273.15;

Pvap = zeros(1, size(T, 2));

for i = 1:size(T, 2)

Pvap(1, i) = p_vap(T(1, i));

end

plot(T, Pvap, 'ro');

hold on;

t = logspace(log(80)/log(10), log(150)/log(10), 300);

p = zeros(1, size(t, 2));

for k = 1:size(t, 2)

sum = 0;

for i = 1:size(T, 2)

product = 1;

for j = 1:size(T, 2)

if j ~= i

product = product*((t(1, k)-T(1, j))/(T(1, i) - T(1, j)));

end

end

sum = sum + product*Pvap(1, i);

end

p(1, k) = sum;

end

plot(t, p);

xlabel('temperature (K)')

ylabel('Vapour Pressure (Pa)')

(d) Fit a polynomial curve to Pvap versus T. Make sure that the polynomial curve fits your data well. Recall that according to the Clapeyron equation

dPvapdT=ΔSΔV=ΔHTΔV.

Calculate ΔH=HV-HL, ΔS=SV-SL and ΔV=VV-VL from the information you have collected so far. For this you can also fit a curve through ΔV(T).

Compare your value of ΔH to the value given in the figure below:

Note that in general the pressure where the transition happens from one phase to another is also called saturation pressure PsatT. For vapor-liquid equilibrium (VLE) PsatT≡Pvap(T).

(e) Plot ΔH and ΔS as a function of temperature.

Plot Of ΔH and ΔS Vs T

Code

Plot Enthalpy (ΔH = HV-HL) and Entropy (ΔS = SV – SL) as a function of Temperature

T = [-125 -130 -140 -150 -160 -170 -175 -180 -183]+273.15;

delH = zeros(1, size(T, 2));

delS = zeros(1, size(T, 2));

for i = 1:size(T, 2)

delH(1, i) = delHS(T(1, i), 2);

delS(1, i) = delHS(T(1, i), 1);

end

plot(T, delS);

xlabel('Temperature K');

ylabel('del S (Sv - Sl) (J/mol)');

figure;

plot(T, delH/1e3);

xlabel('Temperature K');

ylabel('del H (Hv - Hl) (kJ/mol)')

function y = delS(T, h_or_s)

R = 8.314;

vap = p_vap(T);

[A, B] = pt_consts(T, vap);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

check_zr=abs(polyval(PReq,zr))<1.e-8 & zr>1.e-5;

zl = zr(3);

zv = zr(1);

vl = zl*R*T/vap;

vv = zv*R*T/vap;

delT = 5;

if h_or_s == 1

y = (vv-vl)*(p_vap(T + delT) - p_vap(T-delT))/(2*delT); %entropy

else

y = (vv-vl)*T*(p_vap(T + delT) - p_vap(T-delT))/(2*delT); %enthalpy

end

end

(f) Enthalpy calculations:

Choose HIGT=25℃,P=1 bar=0. IG implies ideal gas. This sets our reference state. Now calculate enthalpy as

HT,P-HIGT=25℃,P=1 bar  = HT,P-HIGT,P+HIGT,P-HIGT=25℃,P=1 bar

Here HIGT=25℃,P=1 bar has a value zero therefore it has been struck through.

For Peng-Robinson equation, the departure function for enthalpy or residual enthalpy

HT,P-HIGT,P=RTZ-1+TdadT-a22blnZ+1+2BZ+1-2B.

Also

HIGT,P-HIGT=25℃,P=1 bar=T=298.15 KTCP*dT

where

CP*=25.46+1.519×10-2T-0.7151×10-5T2+1.311×10-9T3 Jmol.K

for oxygen and T is in K.

Obtain the pressure-enthalpy along the isotherm as shown in Figure 6.4-4 in page 3.

Hint:

For saturated liquid, we write

HL(T,PvapT)=HV(T,PvapT)- ΔH(T).

For liquid,

HT,P-HIGT=25℃,P=1 bar  = HT,P-HL(T,PvapT)+HL(T,PvapT)

where

HT,P-HLT,PvapT=RTZ-1+TdadT-a22blnZ+1+2BZ+1-2BT,P-RTZ-1+TdadT-a22blnZ+1+2BZ+1-2BT,PvapT

(g) Entropy calculations:

Choose SIGT=25℃,P=1 bar=0. IG implies ideal gas. This sets our reference state. Now calculate entropy as

ST,P-SIGT=25℃,P=1 bar  =ST,P-SIGT,P+SIGT,P-SIGT=25℃,P=1 bar

Here SIGT=25℃,P=1 bar has a value zero therefore it has been struck through.

For Peng-Robinson equation, the departure function for entropy or the residual entropy

ST,P-SIGT,P=RlnZ-B+dadT22blnZ+1+2BZ+1-2B.

Also

SIGT,P-SIGT=25℃,P=1 bar=T=298.15 KTCP*TdT-RlnP1 bar.

Obtain the pressure-entropy along the isotherm.

Enthalpy Vs Pressure Isotherms

Entropy Vs Pressure Isotherms

(H) Entropy Vs Temperature Isobars

Code

Main

Enthalpy Vs Pressure Isotherms

R = 8.314;

Tc=154.6; %K

Pc=5.046e6; %Pa

Vc=0.0732e-3; %m^3/mol

a=0.45724*(R*Tc)^2/Pc; %at T=Tc

b=0.0778*R*Tc/Pc;

omega=0.021;

T = [-183 -180 -175 -170 -160 -150 -140 -130 -100 -75 -50 -25 0 25 50 75 100 125] + 273.15;

P=logspace(0, log(80)/log(10),300)*1e5; %Pa

nT = size(T, 2);

nP = size(P, 2);

enth = zeros(1, nP);

for t = 1:nT

for p = 1:nP

enth(1, p) = 0.001*enthalpy(T(1, t), P(1, p));

end

plot(enth, P/1e5);

hold on;

end

xlabel('Enthalpy (kJ/mol)');

ylabel('Pressure (bar)');

figure;

Entropy Vs Temperature Isobars

T = 73.15:10:400.15;

P = [1 2 5 10 20 50]*1e5;

nT = size(T, 2);

nP = size(P, 2);

ent = zeros(1, nT);

for p1 = 1:nP

for t1 = 1:nT

ent(1, t1) = entropy(T(1, t1), P(1, p1));

end

plot(ent, T);

hold on;

end

xlabel('Entropy (J/mol)');

ylabel('Temperature (K)');

legend('1 bar', '2 bar', '5 bar', '10 bar', '20 bar', '50 bar');

figure

Entropy Vs Pressure Isotherms

T = [-183 -180 -175 -170 -160 -150 -140 -130 -100 -75 -50 -25 0 25 50 75 100 125] + 273.15;

P=logspace(0,log(80)/log(10),300)*1e5; %Pa

nT = size(T, 2);

nP = size(P, 2);

ent = zeros(1, nP);

for t1 = 1:nT

for p1 = 1:nP

ent(1, p1) = entropy(T(1, t1), P(1, p1));

end

plot(ent, P/1e5);

hold on;

end

xlabel('Entropy (J/mol)');

ylabel('Pressure (bar)');

Enthalpy

function y = enthalpy(T, P)

[A, B] = pt_consts(T, P);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

if T < 173.15

if P< p_vap(T)

Z = max(zr);

elseif P> p_vap(T)

Z = min(zr);

else

return

end

else

Z = PR(T,P);

end

Constants

Pc = 5.046e6;

Tc = 154.6;

R = 8.314;

b = 0.0778*(R*Tc)/Pc;

a = 0.45724*((R*Tc)^2)/Pc;

kappa = 0.4069;

alpha=1.+kappa*(1.-sqrt(T/Tc));

da_dt = (-1)*a*alpha*(kappa/sqrt(T*Tc));

alpha = alpha*alpha;

a=a*alpha;

H(T, P) – HIG(T, P) -

c1 = (T*da_dt - a)/(sqrt(8)*b);

B = (b*P)/(R*T);

c2 = (Z + (1+sqrt(2))*B)/(Z + (1-sqrt(2))*B);

c2 = log(c2);

h1 = R*T*(Z-1) + c1*c2;

HIG(T, P) – HIG(25 C, 1 atm) -

c3 = 25.46*(T-298.15);

c4 = (0.01519/2)*(T^2 - 298.15^2);

c5 = ((0.7151e-5)/3)*(T^3 - 298.15^3);

c6 = ((1.311e-9)/4)*(T^4-298.15^4);

h2 = c3 + c4 - c5 + c6;

Total Enthalpy

y = h1 + h2;

end

Entropy

function y = entropy(T, P)

[A, B] = pt_consts(T, P);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

if T < 173.15

if P< p_vap(T)

Z = max(zr);

elseif P>p_vap(T)

Z = min(zr);

else

return

end

else

Z = PR(T, P);

end

Constants

Pc = 5.046e6;

Tc = 154.6;

R = 8.314;

b = 0.0778*(R*Tc)/Pc;

a = 0.45724*((R*Tc)^2)/Pc;

kappa = 0.4069;

alpha=1.+kappa*(1.-sqrt(T/Tc));

da_dt = (-1)*a*alpha*(kappa/sqrt(T*Tc));

S(T, P ) - SIG(T,P) -

c1 = (da_dt)/(sqrt(8)*b);

B = (b*P)/(R*T);

c2 = (Z + (1+sqrt(2))*B)/(Z + (1-sqrt(2))*B);

c2 = log(c2);

s1 = R*log(Z-B) + c1*c2;

SIG(T, P) - SIG(25 C, 1 atm) -

c3 = 25.46*log(T/298.15);

c4 = (0.01519)*(T - 298.15);

c5 = ((0.7151e-5)/2)*(T^2 - 298.15^2);

c6 = ((1.311e-9)/3)*(T^3-298.15^3);

s2 = c3 + c4 - c5 + c6 - R*log(P/1e5);

Total Entropy -

y = s1 + s2;

end

(I)

Compressibility Factor (Z) Vs Reduced Pressure (Pr)

Code

Pc = 5.046e6;

Tc = 154.6;

R = 8.314;

b = 0.0778*(R*Tc)/Pc;

a = 0.45724*((R*Tc)^2)/Pc;

Pr = logspace(log(0.1)/log(10), log(7)/log(10), 300);

P = Pr*Pc;

Tr = [1 1.1 1.2 1.3 1.5 2];

T = Tr*Tc;

z = zeros(1, size(P, 2));

for i = 1:size(T, 2)

for j = 1:size(P, 2)

[A, B] = pt_consts(T(1, i), P(1, j));

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

%get only relevant values of z

check_zr=abs(polyval(PReq,zr))<1.e-8;

pos=find(check_zr==1);

z(1, j) = zr(pos);

end

plot(Pr, z);

xlabel('Reduced Pressure');

ylabel('Compressiblity Factor Z  (PV/RT)');

legend('Tr = 1', 'Tr = 1.1', 'Tr = 1.2', 'Tr = 1.3', 'Tr = 1.5', 'Tr = 2');

hold on

end

Code that support all the above codes

Function For Calculating Z for Given P and T

function z=PR(T,P)

[A, B] = pt_consts(T, P);

alpha= -1.+B;

beta=A-3*B*B-2*B;

gamma= -A*B + B*B*(B+1.);

PReq=[1.0 alpha beta gamma];

zr=real(roots(PReq));

%get only relevant values of z

check_zr=abs(polyval(PReq,zr))<1.e-8;

pos=find(check_zr==1);

z=zr(pos);

end